Backyard Weather Station Activity – 
Tool #1: Wind Vane Worksheet

Materials

Each group should have:

| - One sheet of cardstock | - One full-sized pencil with full eraser |
| --- | --- |
| - A heavy marker | - One straight pin or T-pin |
| - One pair of scissors | - Small piece of modeling clay |
| - Glue (hot glue works best) | - Piece of cardboard (at least 12” x 12”) |
| - One plastic straw, non-bendable | - A compass |

Instructions

- Push the pin through center of the straw.

- Insert into pencil eraser, making sure the straw has enough room to spin (see Figure at right).

- On a piece of cardstock, draw the shape of an arrow (see below). You can create multiple arrow shapes to test which shape works best in the wind.

- For each arrow design, cut two identical arrow shapes from the cardstock (as shown below).

- Glue one set of arrow shapes onto the sides of the straw so that the straight pin is between the shapes.

- Use a small piece of modeling clay to secure the pencil to the piece of flat cardboard (see picture below. (Note: Although shown on a piece of wood, you will affix the pencil to the cardboard instead).

- Check to make sure that the wind vane will spin and stand straight.

Using the Wind Vane

- Use a compass to figure out which way is North. Mark this direction on the cardboard.

- Let your wind vane blow in the wind. Observe the direction toward which the wind vane points.

- If the tip of the wind vane arrow is narrow enough and the back wide enough, the wind vane should point in the direction the wind is blowing.

- If your wind vane is not giving good results, try designing another arrow shape.

- Use your wind vane to observe the wind direction over a period of time.

- Record your wind data in the table below

Data Collection

Use the following table to collect data from your observations.

| Time of Observation | Wind Direction | Notes |
| --- | --- | --- |