Analysis and Reflection Worksheet Answer Key

### Analyzing and Interpreting Data

### Answer the following questions.

- Looking back at your data, identify how each parameter affected the capacitance. You may use the sentence starters below or write your own on the back.

- When the area of the electrodes increased, the capacitance increased.

- When the distance (separation) between the plates increased, the capacitance decreased.

- When the dielectric material had a higher dielectric constant (permittivity), the capacitance increased.

- Construct an explanation why you think each of these changes had its corresponding effect on capacitance.

- I think increasing the area caused more charge to be stored on the plates, which led to the capacitance increasing.

- I think decreasing the distance between the metal plates caused the electric attraction between opposite charges to increase, which led to the capacitance increasing.

Reflection

### Answer the following question.

- If there were any other things you changed, you can include your explanations below:

Responses will vary.