Part 6B: Arduino Touch-Activated Buzzer Challenge Answers

Example of setup of circuit for touch-activated buzzer with Arduino:

Example of possible code for touch-activated buzzer with Arduino:

// Capacitive Touch Sensor triggers buzzer and LED when pressed

// When Sig Output is high, touch sensor is being pressed and buzzer emits sound

#define

ctsPin 2  // Pin for capacitive touch sensor

int ledPin = 13;  // pin for the LED

const int buzzer = 9;    //pin for buzzer

void setup()

{

Serial.begin(9600);

pinMode(ledPin, OUTPUT);

pinMode(ctsPin, INPUT);

pinMode (buzzer, OUTPUT);

}

void loop()

{

int ctsValue = digitalRead(ctsPin);

if (ctsValue == HIGH)

{

digitalWrite(ledPin, HIGH);

Serial.println("TOUCHED");

tone(buzzer, 1000); // send 1KHz sound signal

delay(1000);        // . . . for 1 sec

noTone(buzzer);     // Stop Sound

delay(1000);        // . . . for 1 sec

}

else{

digitalWrite(ledPin,LOW);

Serial.println("not touched");

}

delay(500);

}