Groundwater Pollution Worksheet

PART 1

- Find the pH at each sample site. Label the concentration as high (H), low (L) or none(N). Draw the direction of the contaminated groundwater flow on the diagram.

| Sample # | pH | Concentration (H, L or N) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |

- Why might the groundwater flow in that direction?

Now, we are going to predict how fast it will take for the contaminant to reach Community A using the diagram below. You will need a ruler.

- First, soil samples were taken at site #4 over one year. On the 10th month, a rise in the level of contaminants was found. Find the velocity of the contaminant at sample site #4. Remember, Velocity (v) = distance (D) ÷ time (t).

- Next, we need to find the area of a cross-section of the contaminant plume at site #4 (shown by the dotted line). Remember Area (A) = width (w) x depth (d). We know that the groundwater is 1 meter in depth.

- We use the cross section of the area we just found to calculate the Flow (Q) of the groundwater over the whole area. Flow (Q) = Area (A) x velocity (v).

- Next, we need to find the area of a cross-section of the plume at community A (shown by the dotted line). Remember Area (A) = width (w) x depth (d). We know that the groundwater is 1 meter in depth.

- Then, we use our flow (Q) from above (assuming the flow is constant) to find the velocity of the contaminant at community A. We rearrange our flow equation to read Velocity (v) = Flow (Q) ÷Area (A).

- Lastly, we rearrange our velocity equation to solve for the amount of time for the contaminant to reach Community A. Our equation now reads time (t) = distance (D) ÷ velocity (v).

- Write your prediction for the time it will take the contaminated groundwater to reach Community A.

Circle the groundwater treatment method you would choose for this site.

| - Treatment Name | - Description | - Time |
| --- | --- | --- |
| - Containment (Physical barriers) | - Placing something in the ground to stop the groundwater flow. | - 1 year |
| - Biological treatment | - Adding microorganisms like bacteria that eat the contaminant to make it less toxic. | - 10+ years |
| - Chemical treatment | - Adding chemicals like that react with the contaminant to make it less toxic. | - 6 months-3 years |
| - Soil vapor extraction | - Moving air and vapors through the groundwater in order to remove the contaminant. | - 3-5 years |
| - Pump and treat | - Pumping the contaminated water out of the ground, treat the water and put it back into the ground. | - 5-10 years |

# PART 2: Remediation

- Using the available tools for water treatment, brainstorm combinations to get your water sample clean. Make a list of your ideas. Be specific.

- Pick one treatment combination from your above list to test. This is your treatment process. Which treatment process did your group choose and why?

- Test your treatment process and write your observations here.

- Was your treatment process effective? Why or why not?

- What improvements would you make to your treatment process?

- Test your modified treatment process from #5 and write your observations here.

- Was your second process more effective than your first process?

- What recommendations would you make for treatment of this contaminated water?

- What constraints did you consider when you designed your treatment process? (Cost, environmental effects, time, etc.)?