# Static Problems Worksheet Answers

- Equilibrium Equations:

 Fx = 0	(1)		 Fy = 0	(2)		 M = 0	(3)

- Determine force F and the reaction force RA that satisfy the equilibrium equations 
for the following beam:

5 m

2 m

F

10 N

A

Step 1.	Draw a free-body diagram of the beam:

F					10 N

ReactionA

Step 2.		Using Equation (3), determine the force F:

ΣMA = F*2 – 10*5 = 0

2F = 50

F = 25 N

Step 3.		Using Equation (2), determine the force RA:

ΣFy = RA – F – 10 = 0

RA = F + 10

RA = 35 N

- Given the diagram below, determine the load CW and RA:

15 m

A

80 N

10 m

WB

10 m

WB = 5 N

CW

Step 1.		Draw a free-body diagram of the entire crane:

CW			      WB			    80 N

ReactionA

Step 2.		Using Equation (3), determine the force CW:

ΣMA = CW*10 – WB*10 – 80*25 = 0

10CW = 10*5 + 80*25

CW = 205 N

Step 3.		Using Equation (2), determine the force RA:

ΣFy = RA – CW – WB – 80 = 0

RA = CW + WB + 80

RA = 290 N