Name:

Class Period:

Date:

Moments and Mechanical Advantage

Needed Supplies: Pressure gauge, ruler

Instructions: Now that you know how cylinders produce force, we will investigate how that force is delivered through the arms of the excavator.

Background

- Moment

  - Moments occur when a force is applied to a point from a distance.

  - M = F x D

    - M is the moment (in – lbf)

    - F is the force (lbf)

    - D is the distance at which the force is applied (in)

  - The moment that Fin creates on the diagram above is:

    - M = (Fin) x (Din)

  - The moment that Fout creates on the diagram above is:

    - M = (Fout) x (Dout)

  - Since Fin and Fout moments act upon the same point (the red triangle), they are equal:

    - (Fin) x (Din) = (Fout) x (Dout)

  - Knowing this, if you apply a force to the lever, Fin, you will produce another force Fout, that is equal to:

    - Fout = [(Fin) x (Din)] / (Dout)

- Mechanical Advantage

  - Mechanical advantage is the ratio of the output force (Fout) to the input force (Fin).

  - MechAdv = Fout / Fin

- Moments and Mechanical Advantage on the excavator

  - The excavator uses the principles of moments and mechanical advantage to do work, with the cylinders providing the force.

  - In the picture below, the top cylinder is extending:

Experiment

- Determine the force produced by the top cylinder when it extends

  - Attach pressure gauge at appropriate port

  - Move the valve that extends the top cylinder, and record the pressure at the gauge

  - P = _________ psi

  - Ac = .88 in2

  - Fin = PAc = (_______psi)(_______in2)

  - Fin = _______lbf

- Measure Din and Dout on the excavator arm

  - Din = ________ in

  - Dout = _________ in

- What is Fout equal to?

  - Fout = [(Fin) x (Din)] / (Dout) = ___________

  - Fout = _________ lbf

- What is the mechanical advantage of the excavator arm?

  - MechAdv = Fout / Fin = ________

  - MechAdv = ________

Discussion

- Is the mechanical advantage different when retracting the cylinder? Explain your answer.

- What would happen to Fout and the MechAdv if Din were decreased?

- Compare the distances that each end of the excavator arm travels. Which travels further when the cylinder extends, the cylinder end, or the bucket end?