Name:

Class Period:

Date:

Bernoulli’s Equation – Energy Conservation

Needed Supplies: Empty 2-liter plastic bottle, scissors, ruler, dye, water

Theoretical Background

- Bernoulli’s Equation

  - An increase in the speed of a fluid occurs simultaneously with a decrease in pressure or a decrease in the fluid’s potential energy.

  - The left side of the equation represents point 1, with the right side representing point 2 (before and after)

    - P1 + 1/2pv21 + pgh1 = P2 +1/2pv22 + pgh2

      - This equation is based on the principle of energy conservation

        - Energy is neither created nor destroyed, but rather changes forms

      - Bernoulli’s equation contains three types of energy:

        - Pressure Energy

          - Represented by 𝑷𝟏 and 𝑷𝟐

        - Kinetic Energy

          - Represented by 1/2 𝝆𝒗𝟐𝟏 and 1/2𝝆𝒗𝟐𝟐

          - 𝝆 is the density of the fluid

          - 𝒗 is the velocity of the fluid

        - Potential Energy

          - Represented by 𝝆𝒈𝒉𝟏 and 𝝆𝒈𝒉𝟐

          - 𝝆 is the density of the fluid

          - g is the acceleration due to gravity

          - h is the height of the fluid from the designated zero point

Experiment

- A 2 Liter soda bottle with a hole will be used for this experiment

- Point 1 – The surface of the water in the bottle

    - 𝑷𝟏 is zero because of atmospheric pressure

    - 𝒗𝟏 is assumed to be zero for this experiment

    - 𝒉𝟏 will be recorded as water level decreases

  - Point 2 – The hole at the bottom of the bottle

    - 𝑷𝟐 is zero because of atmospheric pressure

    - 𝒗𝟐 is unknown but will be calculated

    - 𝒉𝟐 is zero because ℎ1 is measured from point 2

  - Solving the equation for v2

Procedure

- Obtain a 2 liter soda bottle and create a small hole on the bottom side

- Cut off the top of the bottle and insert a ruler

- Fill with water/dye, keeping the hole plugged

- Let water flow out

- Measure the height of the fluid (ℎ1) at 10 different points as the water flows out

- Record results in Data/Calculations section

- Calculate 𝒗𝟐 for each point

Discussion

- Describe what happens to the velocity of water flowing out of the bottle as the water level (h1) gets lower.

- What assumption becomes invalid when the hole size is large enough to make the water at point 1 move with a significant velocity?

- What would happen to 𝒗𝟐 if the top of the bottle is sealed and hooked up to an air compressor?