Name:

Class Period:

Date:

Energy Storage

Needed Supplies: Air compressor, valve, cylinder, piping/tubing, weight, pressure gauge, ruler, stopwatch

Background

  - Air compressors are used every day to do jobs that would otherwise be difficult to do with ordinary hand tools.

  - Air compressors use an air pump to convert mechanical energy to fluid energy, store that energy in a tank, and then expel it through a cylinder or motor, converting it back to mechanical energy.

- Asdf

  - Work

    - Work is defined as force times distance

    - W = FD

      - F in lbs

      - D in inches

  - Power

    - Power is defined as work per unit time

      - Power = Work / t

    - Hydraulic power is the product of Pressure and Flow Rate

      - Power = PQ

        - P in psi

        - Q in in3/sec

  - Efficiency

    - Efficiency is the ratio of power input to power output

      - e = Output power / Input power

Experiment

- In this experiment you will follow the path of compressed air from a tank to a cylinder attached to a weight

- The air within the air compressor tank is stored energy, and creates power when released at a certain pressure (P) and a flow rate (Q).

  - Refer to the specifications of the air compressor model that you are using, specifically what flow rate (in CFM, cubic feet per minute) it can deliver at a certain pressure.

- When the air reaches the cylinder, work is done by the cylinder (a force is applied over a distance)

  - Power is produced during the extension of the cylinder as well (Work/time)

- Between the air tank and the cylinder, power is lost through friction in the pipes as well any leaks in the system

  - It is possible to determine how efficient the system is by dividing the output power at the cylinder by the input power from the compressor tank

Procedure

- Plumb a basic air circuit with 1 valve and 1 cylinder and connect to an air compressor

- Make sure that air compressor has a pressure gauge.

- Attach an object of known weight to the extended cylinder

- Retract the cylinder and record the following:

  - How long it takes to lift the weight

  - The pressure reading from the gauge

  - The distance that the weight travels

Data Calculations

Discussion

- What generally increases with the size of an air compressor tank? (Consult your instructor or online resources if necessary)

- Would decreasing the length of tubing between the tank and cylinder increase or decrease the efficiency of the system? Explain your answer.

- What does an efficiency of 1 mean? Why is the generally not possible in the real world?