# Recommended Insulation Levels for Retrofitting Existing Wood-Framed Buildings

Insulation levels are specified by R-value. R-value is a measure of insulation’s ability to resist heat traveling through it. The higher the R-value the better the thermal performance of the insulation. The table below shows what levels of insulation are cost-effective for different climates and locations in the home.

| Zone | Add Insulation to Attic |  | Floor |
| --- | --- | --- | --- |
|  | Uninsulated Attic | Existing 3–4 Inches of Insulation |  |
| 1 | R30 to R49 | R25 to R30 | R13 |
| 2 | R30 to R60 | R25 to R38 | R13 to R19 |
| 3 | R30 to R60 | R25 to R38 | R19 to R25 |
| 4 | R38 to R60 | R38 | R25 to R30 |
| 5 to 8 | R49 to R60 | R38 to R49 | R25 to R30 |
| Wall insulation: Whenever exterior siding is removed on an uninsulated wood-frame wall: - Drill holes in the sheathing and blow insulation into the empty wall cavity before installing the new siding, and - Zones 3–4: Add R5 insulative wall sheathing beneath the new siding - Zones 5–8: Add R5 to R6 insulative wall sheathing beneath the new siding. Insulated wood-frame walls: - For Zones 4 to 8: Add R5 insulative sheathing before installing the new siding. |  |  |  |

Source: Energy Star, US Department of Energy http://www.energystar.gov/index.cfm?c=home_sealing.hm_improvement_insulation_table