Beam Testing Set-Up and Diagram

Set-Up Supplies

- spring scale or weighing container

- weights (kg or lb) (F)

- 2 table tops (alternative: 2 ring stands)

- 10 lb test fishing line

- fishing line knot with paper towel to avoid line cutting into beam

- composite beams

Set-Up Procedure

  - Cut four- to five-foot length of 10 lb test fishing line (A).

  - Fold line length in half and fasten to spring scale or weighing container (B, A).

  - Tie the ends together and wrap a paper towel in a neat folded fashion; place the towel on the beam top to protect the beam (see C); the large loop should have two tension lines per side (see A).

  - Move together two sturdy tables (see D) (or ring stands), leaving a distance between them of approximately three inches.

  - Make sure the beam (E), string and pan height have a clearance from the ground (or table top) of a minimum ¼ inch.

kg

B

A

C

D

D

F

E

minimum ¼ in

3.0 in