Mold Construction Instructions

Materials List

- 9-in x 12-in sheet of aluminum foil

- masking tape

- scissors

- ruler

| 12-in 9-in Start with a 9 x 12-in foil sheet  | seam Fold in half with seam along 12-in direction |
| --- | --- |
| seam 2” Fold top and bottom edges 2 inches towards center with both fold seams along 9-in direction | 1.5” Fold left and right sides 1.5-in in towards center |
| 0.5 in Rotate foil 90 degrees and fold top and bottom edges 0.5-in towards center  | Flip foil over and fold top and bottom sides up forming a U shape with 0.5-in wide base |
| 1.5 in Cut base on both sides of left and right sides 1.5 in in towards center | fold 1 in Fold both middle tabs way from mold center 1.0 in |
| 0.5 in Cut outer tabs down to 0.5-in on both sides | Fold outer tabs behind middle tab forming end mold wall; repeat on other end |
| Take 1.5-in strip of masking tape and tape wrapped mold side walls to mold wall end  | Place 1-in strip of masking tape so half is on mold bottom and the other half is on the end mold wall |
| Final beam mold! Final mold dimension = 0.5 x 0.5 x 5 inches (1.27 x 1.27 x 12.7 cm) (NOTE: Adjust slightly, as needed, to make dimensions as close to final beam dimensions as possible) |  |