Material Science Quiz Answers

Answer the following questions without the assistance of a neighbor, friend or teacher.

- For each material, indicate its material class.

| quartz | 3 | Material Classes: |
| --- | --- | --- |
| steel | 2 | 1. polymer |
| Teflon® | 1 | 2. metal |
| aluminum | 2 | 3. ceramic |
| plywood | 4 | 4. composite |
| fiber glass | 4 |  |
| diamond | 3 |  |
| Zip Lock® bag material | 1 |  |
| paper clip | 2 |  |
| polyvinyl chloride (PVC) | 1 |  |
| concrete | 4 |  |
| paper | 4 |  |

- Connect each material to the corresponding material class.

| ductility | 8 | 1. Hard, covalently bonded material |
| --- | --- | --- |
| brittle | 9 | 2, Describes displacement of particles in a deforming body |
| lattice | 7 | 3. Measured stress at onset of plastic deformation |
| ceramic | 1 | 4. The brand of science that deals with metal properties |
| stress | 10 | 5. Irreversible alteration of a solid body under stress |
| plastic deformation | 5 | 6. Reversible alteration of a solid body under stress I |
| yield strength | 3 | 7. 3D geometric arrangement of atoms composing a crystal |
| elastic deformation | 6 | 8. Ability of material to undergo permanent deformation without fracture |
| strain | 2 | 9. Ability of material to break, or crack easily when subjected to a force |
| metallurgy | 4 | 10. Force exerted on an object over a defined cross-sectional area |

- What type of bonding makes up ceramic materials?

  - covalent bonds

  - London dispersion forces

  - metallic bonds

  - ionic bonds

- How many lattice structures exist?

  - 2

  - 14

  - 7

  - 5

- How many Bravais lattice structures exist?

  - 2

  - 14

  - 7

  - 5

- The maximum attainable stress for a metal is called:

  - yield stress

  - fracture stress

  - maximum stress

  - ultimate tensile stress

- All are attributes of ceramics, except:

  - covalent bonded

  - low melting point

  - high stiffness

  - High hardness

- All are not attributes of metals, except:

  - electrical insulators

  - thermal insulators

  - high melting points

  - ductile

- Below are examples of plastic deformation, except:

  - a wire coiled 10 times around a magnetic core

  - broken glass

  - bent nail in wood

  - molded clay

- What class of material might be useful at temperatures 1100°C (2150°F)?

  - pure metals

  - polymer-polymer composites

  - ceramics-ceramic composites

  - lead-tin metal alloy

- Label the stress strain plot below:

  - yield stress

  - ultimate tensile stress

a

b

c

d

e

fracture stress

  - elastic deformation region

  - plastic deformation region

- Label the lattice and packing below.

cubic; primitive

cubic; face centered (FC)

cubic; body centered (BC)

- True or False? 	Aluminum alloy has an FCC structure.

- True or False? 	Brittle materials typically fail fast and with excessive plastic deformation.

- True or False? 	Strain is the measure of force per unit cross-sectional area.

- True or False? 	Type of bonding dictates whether a material is electrically conductive or not.

- True or False? 	Metallurgy is the study of metals and their behavior.

- True or False? 	I like materials science and engineering.