Designing and Building a Robotic Hand Using
the Force Sensor Circuit GUIDANCE & TIPS

Use this instruction manual to help you design and build a robotic hand. Incorporate the force sensor into the inside of a glove finger. The sensor remains connected to the electric circuit as force is applied to an object. Make sure you have the following items:

robotic hand circuit with sensor

wood puck

threaded needle

super glue

glove

NOTE: Record the minimum force required to crack an egg on the Building an Electric Circuit Evaluation Sheet.

- FlexiForce Sensor

  - Disconnect ONLY the FlexiForce sensor 
from the robotic hand circuit.

  - You may want to replace some sensor 
wires with longer ones.

  - The sensor will be used inside the glove!!

Figure 1

- Glove Stitching Removal

  - Turn glove inside out.

  - Choose any glove finger and remove the stitching from the upper half.

Figure 2A

Figure 2B

- Sensor

  - Turn glove inside out.

  - Place the FlexiForce sensor inside the chosen glove finger.

Tip: The sensor sensing area should be facing DOWN.

  - Secure the sensor inside by sewing around it from side to side.

Tip: Make sure not to pierce the sensor!

Figure 3A

Figure 3B

- Puck

  - Glue the puck to the sensor sensing area.

The puck is between the sensing area and the glove surface.

  - Sew the glove finger back to its original state.

Figure 4B

Figure 4A

- Glove Testing

  - Connect the glove to the robotic hand circuit.

  - Connect the multimeter, if it is not already connected.

  - Add pressure to an object (the egg).

Figure 5B

Figure 5A